# Course Notes: Robotic System Components & Joints

# 1. Core Components of a Robotic System

A standard robot consists of a base, links, and joints, along with the following key sub-systems:

End-Effector (Gripper): Attached to the wrist joint (the last link of the robot). Its purpose is to grip and manipulate objects.

Drive System (Actuator): Provides power for movement, similar to human muscles (mechanical) or the heart/blood (hydraulic). Types include:

Mechanical drives (gears, pinions, chains, belts)

Hydraulic drives (used for high load/power requirements)

Pneumatic drives (uses compressed air)

Electrical & Combination drives (electro-hydraulic, electro-pneumatic)

Controller (The Brain): Contains the hardware and software required to process information, make decisions, and instruct the drive system.

Sensors: Collects data to make the robot intelligent (mimicking human senses like eyes/skin).

Internal Sensors: Monitor the robot's own state to operate drive units (e.g., position, velocity, acceleration, and force/moment sensors).

External Sensors: Collect information about the environment (e.g., range and proximity sensors).

# 2. Multidisciplinary Areas in Robotics

Robotics relies heavily on a foundation of Mathematics and Physics, divided into distinct modules based on engineering disciplines:

| Discipline | Robotics Application / Module |
| --- | --- |
| Mechanical Engineering | • Kinematics: Studies relative motion of joints/links without considering forces. • Dynamics: Determines the forces (linear) or torques/moments (rotary) required for movement. • Sensing: Hardware for environment data collection. |
| Computer Science | • Motion Planning: Algorithms (traditional or soft computing) to decide the robot's course of action. • Artificial Intelligence (AI): Modeling human intelligence to give the robot a "brain" (traditional AI vs. non-traditional computational intelligence). |
| Electrical/Electronics Engineering | • Control Schemes: Efficient control of joint motors (typically DC motors) and the physical hardware implementation of controllers. |

Note: To be a true roboticist, one must have fundamental knowledge across all these pillars.

# 3. Robotic Joints & Degrees of Freedom (DoF)

Definition of Connectivity (DoF): The number of independent movements an output link has relative to an input link, or how many rigid links can be connected to one fixed link through a joint.

## 1-Degree of Freedom Joints

Linear Joints:

Prismatic Joint (P): Sliding motion (e.g., a block moving up/down). Joint angle is fixed, movement is strictly linear.

Sliding Joint (S): Similar strictly linear movement.

Rotary Joints:

Revolute Joint (R): Axis of rotation is at 90 degrees to the axis of the output link. (Example: Tilting your head up and down). The variable is the joint angle (θ).

Twisting Joint (T): Axis of rotation coincides with the axis of the output link. (Example: Shaking your head left and right).

## Multi-Degree of Freedom Joints

Cylindrical Joint (2 DoF): A combination of one linear and one rotary movement. The link can slide up/down and rotate. Variables: θ_j (rotation) and d_j (linear displacement).

Hooke / Universal Joint (2 DoF): Formed by combining two revolute joints. Commonly used in parallel manipulators, not serial.

Spherical / Ball and Socket Joint (3 DoF): Used exclusively in parallel manipulators. Allows rotation across 3 axes (X, Y, Z).

Transformation Mechanics: Starting from input link coordinates (X1, Y1, Z1) to reach output coordinates (X2, Y2, Z2) requires 3 distinct rotations:

1. Rotation about Z by angle α (alpha)

2. Rotation about X by angle β (beta)

3. Rotation about Y by angle γ (gamma)

# 4. Serial vs. Parallel Manipulators

Serial Manipulators: All links and joints are connected in a single series (e.g., a TRS manipulator - Twisting, Revolute, Sliding joints in a row). Typically use 1 DoF joints.

Parallel Manipulators: Feature closed-loop chains. Examples include a top plate supported by multiple independent legs, each containing multiple joints. Often utilize complex joints like Hooke or Spherical joints that are not used in serial arms.