# Course Notes: Manipulator Mobility, Classification & Workspace

# 1. Grubler's Criterion: Stewart Platform Example

A Stewart Platform is a complex spatial parallel manipulator (often used for aircraft trainee pilots) consisting of a fixed base plate and a moving top plate connected by 6 independent legs.

## Structure per Leg:

1 Universal (Hooke) Joint (2 DoF) -> Imposes 4 constraints (6 - 2 = 4)

1 Prismatic Joint (1 DoF) -> Imposes 5 constraints (6 - 1 = 5)

1 Spherical Joint (3 DoF) -> Imposes 3 constraints (6 - 3 = 3)

Total constraints per leg: 4 + 5 + 3 = 12

## Mobility Calculation:

Number of moving links (n): 6 legs × 2 links/leg + 1 top plate = 13

Total constraints (Σ(6 - Ci)): 6 legs × 12 constraints/leg = 72

M = 6n - Σ(6 - Ci)
M = 6(13) - 72
M = 78 - 72 = 6

Conclusion: The Stewart Platform is an ideal parallel spatial manipulator with 6 degrees of freedom (3 translations, 3 rotations).

# 2. Classification of Robots

Robots can be classified using several different criteria:

## A. Based on Task Type

Point-to-Point Robot: The tool is withdrawn from the job between tasks. (e.g., Drilling holes at specified locations). Examples: Unimate 2000, T3.

Continuous Path Robot: The tool remains continuously in touch with the job. (e.g., Tracing a complex profile with a milling cutter). Examples: PUMA, CRS.

Note: A continuous path robot can perform point-to-point tasks, but the reverse is not true.

## B. Based on Controller Type

Non-Servo-Controlled Robots: Uses an open-loop control system (no feedback loop). Errors are not measured or compensated for. Less accurate, less expensive. Example: Seiko PN-100.

Servo-Controlled Robots: Uses a closed-loop control system. Output is measured, compared to the input, and the resulting error is fed back into the controller to minimize error. Highly accurate, more expensive. Examples: Unimate 2000, PUMA, T3.

## C. Based on Coordinate System

Cartesian Coordinate Robot (PPP or SSS): Features 3 independent linear movements (X, Y, Z). Highly rigid and accurate, ideal for pick-and-place floor operations. Examples: IBM RS-1, Sigma (Olivetti).

Cylindrical Coordinate Robot (TSS or TPP): Features 2 linear joints and 1 rotary joint. Limitations include restricted vertical/horizontal reach near the base and poor dynamic performance due to the rotary joint. Example: Versatran 600.

Spherical / Polar Coordinate Robot (TRS or TRP): Features 1 linear joint and 2 rotary joints. Useful for picking objects off the floor, but shares dynamic performance limitations. Example: Unimate 2000B.

Revolute / Articulated Robot (TRR): Features 3 rotary joints. Highly versatile and widely used in industry for drilling, milling, etc. Examples: PUMA, T3, CRS.

## D. Based on Mobility Levels

Robots with Fixed Base (Manipulators): Can be serial (links in series) or parallel (links in parallel, offering higher load-carrying capacity).

Robots with Moving Base (Mobile Robots): Wheeled (for smooth terrain), Tracked (for moderate terrain), or Multi-legged like 4-leg or 6-leg (for highly uneven, rough terrain).

# 3. Workspace of a Manipulator

Definition: The total volume of space that the end-effector of a manipulator can physically reach.

Reachable Workspace: The volume of space the end-effector can reach with at least one orientation (typically the outer boundary of its maximum reach).

Dextrous Workspace: The volume of space the end-effector can reach with multiple different combinations of joint angles (different orientations).

Key Rule: The dextrous workspace is always a smaller subset of the reachable workspace.