# Course Notes: End-Effectors & Grippers

# 1. What is an End-Effector?

The end-effector is attached to the wrist joint (the final joint) of a robot. Its primary purpose is to hold materials, handle parts, or manipulate tools to perform specific tasks.

## Types of End-Effectors:

Grippers: Used to grasp objects, parts, or materials.

Specialized Tools: Used when the robot acts as the operator. Examples include welding electrodes or spray painting guns.

# 2. Classification of Grippers

## A. Single vs. Double Gripper

A single gripper features one independent grasping unit, while a double gripper utilizes two independent grasping mechanisms attached to the wrist.

## B. Internal vs. External Gripper

Figure 1: Internal vs External gripping mechanisms on a hollow pipe.

Internal Gripper: Grips an object (like a hollow pipe) by pressing its pads outward from the inside.

External Gripper: Grips an object by clamping down on the outside surface.

## C. Soft vs. Hard Gripper (Force vs. Form Closure)

Hard Gripper: Uses a point contact to grip an object. Relies heavily on Force Closure (applying strict pressure to prevent slipping, like gripping a perfectly round chalk).

Soft Gripper: Uses area contact to grip an object. Often relies on Form Closure, where the physical geometry of the object (e.g., the corners of a square chalk) helps secure the grip with less applied force.

## D. Active vs. Passive Gripper

Active Gripper: Equipped with sensors (like touch sensors in human skin) to provide feedback during grasping.

Passive Gripper: A purely mechanical gripper without any sensory feedback.

# 3. Simple Mechanical Gripper Designs

Mechanical grippers are cost-effective but less versatile. They usually rely on a sliding piston-cylinder to actuate the fingers.

Linkage Actuation: The sliding piston connects to linkages that alter the joint angle (theta) to pinch or release the jaws.

Swing-Block Mechanism: A piston drives a link through circular grooves in two swing-blocks. As the piston slides forward, the blocks are forced together.

Rack and Pinion Mechanism: A small rotating gear (pinion) drives two opposed racks connected to the gripper pads. Clockwise rotation closes the grip, while counter-clockwise opens it.

Cam and Follower Mechanism: The piston drives a cam profile against roller followers. Moving the cam forces the rollers apart (opening the grip) or together (closing the grip).

# 4. Vacuum Grippers

Vacuum grippers are ideal for handling large, flat objects like sheet metal or glass. They consist of elastic cups (made of rubber/soft plastic for hard objects, or hard materials for soft objects) attached to the robot's wrist.

## Working Principle (Bernoulli’s Equation):

Air is forced through an orifice plate (a plate with small holes), and then into a venturi tube.

According to the continuity equation, forcing air through a smaller cross-sectional area causes its velocity to increase.

According to Bernoulli's equation, as velocity increases, pressure decreases.

This pressure drop creates a vacuum (below atmospheric pressure) inside the elastic cup.

Because the outside air is at normal atmospheric pressure, the resulting pressure differential holds the flat object securely against the cup.

To un-grip, the airflow is simply stopped. The pressure inside the cup normalizes with the atmosphere, and the object releases due to gravity.