# Course Notes: Inertia of Circular Links & Introduction to Lagrange-Euler Formulation

# 1. Inertia Tensor for a Circular Cross-Section Link

Previously, we calculated the inertia tensor for a rectangular link. Now, we examine a robotic link with a solid circular cross-section of radius 'r' and length 'l', where the coordinate frame is attached at the motor end.

## Setting up the Integration:

To integrate over the volume of a cylinder, we use cylindrical coordinates:

y = r * sin(θ)

z = r * cos(θ)

Differential Volume (dv) = (r * dθ * dr) * dx

Differential Mass (dm) = density (ρ) * r * dθ * dr * dx

## Integration Limits:

θ: 0 to 2π

r: 0 to R (outer radius)

x: -l to 0 (origin is at the motor end, so the link extends into the negative X direction)

## Moments of Inertia (Results):

By solving the triple integrals, we find the primary moments of inertia:

I_xx = (1/2) * m * r²

I_yy = (m * l² / 3) + (m * r² / 4)

I_zz = (m * l² / 3) + (m * r² / 4)

Because the circular link is perfectly symmetrical, all Products of Inertia (I_xy, I_yz, I_zx) integrate to 0.

## Center of Mass:

Integrating 'x * dm' yields (-l/2, 0, 0), proving the center of mass is halfway down the length of the link.

## The Slender Link Approximation (Circular):

If the length 'l' is significantly larger than the radius 'r', we approximate 'r' as approaching 0. This removes the 'mr²/4' terms from the matrix, simplifying dynamic calculations.

# 2. Real-World Note on Link Geometry

In actual industrial robots, the physical links rarely have a constant, perfect rectangular or circular cross-section. The geometry varies along the length of the arm (just like a human arm tapers from bicep to wrist).

Because manual integration is practically impossible for complex, varying geometry, engineers calculate the inertia tensor of real robotic links using Finite Element Analysis (FEA) software.

# 3. The Lagrange-Euler Formulation

To find the exact torque/force (τ) required to move a joint, we use the Lagrange-Euler energy formulation. This method calculates torque based on the entire system's kinetic and potential energy.

## The Lagrange Equation:

τ_i = (d/dt)[ ∂L / ∂(q'_i) ] - (∂L / ∂q_i)

τ_i: Generalized torque/force required at joint 'i'.

q_i: Generalized coordinate (Joint angle θ for revolute, Offset d for prismatic).

q'_i: First time derivative of q_i (Velocity).

L (Lagrangian): Total Kinetic Energy (K) minus Total Potential Energy (P). [L = K - P]

# 4. Calculating Kinetic Energy for a Link

The kinetic energy (K) of a moving particle is (1/2 * m * V²). To find 'V', we must find the derivative of the particle's position with respect to time.

## Rigid vs. Flexible Links:

For a purely Rigid Link, the position of a particle relative to its own coordinate frame does not change over time. Therefore, its local derivative is 0, simplifying the math.

For a Flexible Link, the local position changes as the arm bends. This creates non-zero local derivatives, requiring complex FEA to solve.

## Trace Notation for V²:

Squaring a 3D velocity vector [vx, vy, vz] yields a matrix. The sum of the squares (vx² + vy² + vz²) lies along the diagonal of this matrix. In linear algebra, the sum of the diagonal elements is called the 'Trace'.

Therefore, the Kinetic Energy of the differential mass is written mathematically using the Trace operator:

dK_i = (1/2) * Trace( V_i * V_i^T ) * dm