# Course Notes: Robot Dynamics & Lagrange-Euler Formulation

# 1. Kinetic Energy of the Manipulator

The total kinetic energy (K) for the entire manipulator is the sum of the kinetic energies of each individual link (k_i).

## Kinetic Energy of the i-th Link:

k_i = (1/2) * Trace[ Σ(a=1 to i) Σ(b=1 to i) U_ia * J_i * U_ib^T * q'_a * q'_b ]

## Total Kinetic Energy (K):

By summing the energy of all 'n' links, we get the total kinetic energy:

K = (1/2) * Σ(i=1 to n) Σ(a=1 to i) Σ(b=1 to i) Trace( U_ia * J_i * U_ib^T ) * q'_a * q'_b

# 2. Potential Energy of the Manipulator

The potential energy depends on the mass of the links and the acceleration due to gravity.

## Gravity Vector Padding:

Gravity is typically considered only along the Z-axis (acting vertically downward), making the X and Y components negligible.

To allow matrix multiplication with the 4x1 position vector, the 1x3 gravity vector [gx, gy, gz] is padded with a zero to become a 1x4 matrix: [gx, gy, gz, 0].

## Total Potential Energy (P):

P = Σ(i=1 to n) P_i

P = Σ(i=1 to n) [ -m_i * g_bar * T_i^0 * r_i^i ]

# 3. The Lagrangian (L)

The Lagrangian of the robotic system is simply the difference between the total kinetic energy and the total potential energy.

L = K - P

# 4. Generalized Joint Torque Formulation

By inserting the Lagrangian (L) into the Lagrange-Euler equation, we derive the generalized torque/force required at a specific joint.

## The Lagrange-Euler Equation:

τ_i = (d/dt)[ ∂L / ∂(q'_i) ] - (∂L / ∂q_i)

## Torque Components:

The resulting equation for joint torque (τ_i) consists of three distinct physical components:

1. Inertia Term (D_ic): Depends on the mass distribution of the links.

D_ic = Σ(j = max(i,c) to n) Trace( U_jc * J_j * U_ji^T )

2. Coriolis and Centrifugal Term (h_icd): Dynamic forces generated during multi-joint movement.

h_icd = Σ(j = max(i,c,d) to n) Trace( U_jcd * J_j * U_ji^T )

3. Gravity Term (C_i): The torque required to hold or move the link against the downward pull of gravity.

C_i = Σ(j=i to n) [ -m_j * g_bar * U_ji * r_j^j ]