# Course Notes: Joint Torque Calculation (2-DoF Manipulator)

# 1. Problem Statement

We are analyzing a 2-Degree-of-Freedom serial manipulator operating in a 2D plane. The arm has two links (lengths L1, L2) and two joints (angles θ1, θ2).

The Goal: To use the Lagrange-Euler formulation to derive the exact mathematical expressions for the joint torques (τ1 and τ2) required to move the robot. This is an Inverse Dynamics problem.

# 2. Expanding the Torque Equations

Using the generalized Lagrange-Euler torque equation, we expand the terms for both joints (i=1 and i=2).

## Equation for Joint 1 (τ1):

τ1 = D_11*θ1'' + D_12*θ2'' + h_111*(θ1')² + h_112*(θ1'*θ2') + h_121*(θ1'*θ2') + h_122*(θ2')² + C_1

## Equation for Joint 2 (τ2):

τ2 = D_21*θ1'' + D_22*θ2'' + h_211*(θ1')² + h_212*(θ1'*θ2') + h_221*(θ1'*θ2') + h_222*(θ2')² + C_2

To solve these, we must systematically calculate every individual 'D' (Inertia) term and 'h' (Coriolis/Centrifugal) term using the partial derivatives of our Transformation matrices.

# 3. Determining the Partial Derivative Matrices (U)

The 'U' matrices represent the rate of change of a specific transformation matrix with respect to a specific joint angle.

U_ij = ∂(T_i^0) / ∂(θ_j)

For example, to find U_11, we take the 4x4 transformation matrix T_1^0 (derived from forward kinematics) and calculate the partial derivative of every single element inside it with respect to θ1.

By doing this systematically, we derive the four base U matrices: U_11, U_21, U_22 (Note: U_12 is 0 because T_1^0 does not contain θ2).

# 4. Calculating the Inertia Terms (D)

The 'D' terms are calculated using the 'U' matrices, the Inertia Tensors (J1, J2), and the Trace operator.

## Example: Calculating D_11

Using the general formula: D_ic = Σ Trace( U_jc * J_j * U_ji^T )

D_11 = Trace( U_11 * J1 * U_11^T ) + Trace( U_21 * J2 * U_21^T )

Process:

1. Multiply the three 4x4 matrices (U * J * U^T) for the first term.

2. Multiply the three 4x4 matrices for the second term.

3. Find the Trace (sum of the diagonal elements) for both resulting matrices.

4. Add the two traces together to get the final scalar expression for D_11.

This same matrix multiplication process is repeated to find D_12, D_21, and D_22. (Note: Mathematically, D_12 will always equal D_21).

# 5. Calculating the Coriolis & Centrifugal Terms (h)

The 'h' terms require finding the second-order partial derivatives, denoted as U_ijk:

U_ijk = ∂(U_ij) / ∂(θ_k)

For example, to calculate U_111, we take the U_11 matrix and differentiate it again with respect to θ1.

## Example: Calculating h_111

Using the general formula: h_icd = Σ Trace( U_jcd * J_j * U_ji^T )

h_111 = Trace( U_111 * J1 * U_11^T ) + Trace( U_211 * J2 * U_21^T )

After multiplying the 4x4 matrices and taking the traces, we find that for this specific manipulator, h_111 equates perfectly to 0.

This tedious matrix multiplication and trace calculation must be repeated for all 8 'h' terms (h_111, h_112, h_121, h_122, h_211, h_212, h_221, h_222) to complete the torque equations.