# Course Notes: Calculating the 'h' and 'C' Terms for Joint Torque

# 1. Deriving the Coriolis & Centrifugal (h) Terms

In the previous section, we established that there are 8 'h' terms required for the 2-DoF serial manipulator's torque equations. These terms describe the dynamic forces occurring during multi-joint movement.

## The Methodology:

The formula for an individual 'h' term is:

h_icd = Σ Trace( U_jcd * J_j * U_ji^T )

To solve for a specific term, like h_211:

1. Determine the summation limit (j): The variable 'j' is the maximum of (i, c, d). For h_211, the maximum of (2, 1, 1) is 2. Therefore, 'j' equals 2, meaning there is only ONE term in this specific summation.

2. Determine the U_jcd matrix: For h_211, this requires U_211. U_211 is simply the partial derivative of U_21 with respect to θ1.

3. Multiply the three resulting 4x4 matrices (U_211 * J2 * U_22^T) and calculate the trace.

## Results for the 2-DoF Arm:

By systematically applying these partial derivatives, matrix multiplications, and trace calculations, we find:

h_212 = 0

h_221 = 0

h_222 = 0

# 2. Deriving the Gravity (C) Terms

The 'C' terms calculate the torque required at a specific joint to hold or move the link against the downward pull of gravity.

## The Gravity Vector:

Gravity acts vertically downward, opposite to the standard Y-axis direction. Therefore, the gravity vector is defined as [0, -g, 0, 0]. The extra zero is added specifically to pad the vector into a 1x4 matrix, allowing it to be mathematically multiplied against the 4x1 position vector generated by the forward kinematics equations.

## The Methodology (Example C1):

The formula for a gravity term is:

C_i = Σ [ -m_j * g_bar * U_ji * r_j^j ]

For C1 (the gravity torque on Joint 1), the summation spans from j=1 to j=2 (since both links affect the base joint).

C1 = (-m1 * g_bar * U_11 * r_1^1) + (-m2 * g_bar * U_21 * r_2^2)

The position vector of the center of mass for a uniform link (r_1^1) is located exactly halfway down its length along the negative Y-axis: [-L1/2, 0, 0, 1]^T.

# 3. The Final Torque Equations (τ1 and τ2)

By compiling all the D, h, and C terms, we arrive at the massive, complex algebraic equations defining the total torque required at Joint 1 and Joint 2.

## Key Observations from the Final Equations:

Dynamic Coupling: The torque required at Joint 1 (τ1) is heavily influenced by the position, velocity, and acceleration of Joint 2 (θ2). Likewise, τ2 is influenced by θ1. This phenomenon is known as 'Dynamic Coupling'. Moving one joint physically impacts the forces felt by the other.

The Slender Link Approximation: The final equations contain terms multiplied by the radius squared (r²). If we assume the links are 'slender' (length is vastly greater than the radius), we can approximate r as 0. This mathematically eliminates the (r²) terms, drastically simplifying the torque equations for real-time control software.

Structured Formulation: While the Lagrange-Euler method requires tedious, extensive matrix calculus, it provides a highly structured, organized final equation where the distinct physical effects (Inertia, Coriolis, Gravity) are clearly isolated into separate terms.