# Course Notes: Alternate Lagrange Approach, Control Schemes & Intelligence

# 1. Alternate Lagrange-Euler Approach (Center of Mass)

Previously, dynamics were calculated by attaching the coordinate frame to the base of the link (motor location). An alternative is attaching the coordinate system directly to the Center of Mass (C).

## Parallel Axis Theorem:

If we use the Center of Mass, we recalculate moments of inertia using the Parallel Axis Theorem:

I_ZZ(c) = I_ZZ(base) - m*(X_bar² + Y_bar²)

For a uniform circular link, the new moment of inertia simplifies to: I_ZZ(c) = (1/12)*m*L² + (1/4)*m*r²

## Kinetic and Potential Energy:

Kinetic Energy (K) is the sum of linear (1/2*m*v²) and rotational (1/2*I*ω²) energies.

Potential Energy (P) is based on the vertical height of the center of mass (e.g., P1 = -m1*g*(L1/2)*sin(θ1)).

# 2. Robot Control Schemes

Once the required joint torque (τ) is mathematically determined, a motor must physically generate it. DC motors are commonly used, where the generated torque (τ_m) is proportional to the armature current (I_A).

τ_m = K_m * I_A (where K_m is the motor constant)

## Partitioned Control Scheme:

This scheme divides the calculated torque into two parts: τ = α*τ' + β

α = D(θ) [The Inertia terms]

β = h(θ, θ') + C(θ) + F(θ, θ') [Coriolis, Centrifugal, Gravity, and Friction terms]

τ' is the commanded acceleration generated by the controller.

## Controller Types:

PD Controller (Proportional-Derivative): τ' = θ''_d + K_P*E + K_D*E'

PID Controller (Proportional-Integral-Derivative): τ' = θ''_d + K_P*E + K_I*∫Edt + K_D*E'

Where E is the error (θ_desired - θ_actual). Gain values (K_P, K_I, K_D) can be tuned using the Ziegler-Nichols rule.

# 3. Control Architecture & The PUMA Robot

Robotic control systems use closed-loop feedback (e.g., optical encoders) to continuously measure actual joint angles and velocities, compare them to desired values, and adjust the motor current to compensate for errors.

PUMA Control Architecture: For a 6-DoF robot like the PUMA, each of the 6 joints has its own dedicated control loop with a microprocessor, digital-to-analog converter (DAC), and current amplifier. A central 'Master Control Computer' oversees and coordinates all individual joint controllers.

# 4. Introduction to Intelligent & Autonomous Robots

Up to this point, the robot is a mechanical executor. The final tier of robotics involves decision-making.

## Definitions:

Intelligent Robot: A robot capable of analyzing its environment (via sensors/cameras) and determining the correct course of action in varying situations.

Autonomous Robot: A robot that not only has the intelligence to make decisions but also possesses the ultimate authority/permission to execute those decisions independently.

Note: All autonomous robots are intelligent, but not all intelligent robots are autonomous. (e.g., A smart college faculty is intelligent, but if they must wait for university permission, they are not autonomous).