# Course Notes: Sensors Part 2 - Encoders, LVDT & Force Sensors

# 1. Incremental Optical Encoders

Unlike absolute encoders (which need a photo-detector for every concentric ring), incremental encoders use only ONE coded disc and TWO fixed photo-detectors (A and B).

The disc has alternating black (dark) and white (light) zones.

Direction of Rotation: By observing which detector enters the dark zone first, the direction can be determined. For example, if A enters the dark zone before B, the shaft is rotating clockwise. If B enters first, it is rotating counter-clockwise.

Angular Displacement: Determined by counting the total number of light and dark zones passed.

Advantage: Much less costly than absolute encoders, making it a very common position feedback device in robotics.

# 2. Linear Variable Differential Transformer (LVDT)

Used to measure linear displacement (d). Note: The rotary equivalent is the RVDT for angular displacement (θ).

Construction: Consists of a fixed casing, a moving magnetic core, one primary coil (L_p), and two pairs of secondary coils (L_s1 and L_s2).

Working Principle: An input voltage (V_in) is applied to the primary coil. As the magnetic core slides, it changes the magnetic coupling with the secondary coils.

If the core moves closer to L_s2, the coupling is stronger, making the induced voltage V_L_s2 greater than V_L_s1, yielding a positive output voltage (V_out).

If the core moves closer to L_s1, V_out becomes negative.

By mapping V_out against the core position, a calibration curve is established to accurately measure displacement.

# 3. Force and Moment Sensors

Used at the robot's wrist joint to measure the forces and torques applied during manipulation.

Construction: The sensor has a 'rim' connected to the wrist and a 'hub' connected to the end-effector. Between them are four elastic deflection bars (usually steel with a square cross-section).

Strain Gauges: Two pairs of strain gauges are mounted on each of the four bars (yielding 8 pairs total).

Measuring Deflection: The strain gauges act as a potentiometer circuit to measure the physical deflection (δ) of the bars.

Using the cantilever beam formula within the elastic limit [δ = (P * L³) / (3 * E * I)], the exact load (P) causing the deflection is calculated.

The Calibration Matrix (C_M): The 8 raw readings (W_1 to W_8) from the strain gauges form an 8x1 matrix. To convert this into the 6 actual force/moment components (F_x, F_y, F_z, M_x, M_y, M_z), the readings are multiplied by a 6x8 calibration matrix (C_M).

Precautions:

1. Strain gauges must be perfectly mounted with no gaps.

2. The deflection bars must operate strictly within their elastic limits for the deflection formulas to remain valid.