# Course Notes: Rotation Matrices & Coordinate System Transformations

# 1. Properties of Rotation Matrices

A standard 3x3 rotation matrix possesses several fundamental mathematical properties:

Unit Vectors: Each row or column of a rotation matrix represents a unit vector. The square root of the sum of the squares of its elements equals 1.

Orthogonality (Inner Product = 0): The dot product (inner product) of any row with any other row (or column with another column) is exactly 0. This indicates the axes are mutually perpendicular.

Non-Commutative: The order of rotation operations matters greatly. Rotating about X then Y is NOT equal to rotating about Y then X. Sequence dictates the final orientation.

Inverse equals Transpose: For a pure rotation matrix, its mathematical inverse is identical to its transpose. Note: This is ONLY true for pure rotation, not for transformation matrices containing translation.

R_inverse = R_transpose

T_B_with_respect_to_A = Inverse( T_A_with_respect_to_B )

# 2. The Composite Rotation Matrix Rule

Because rotation matrices are non-commutative, the robotics community follows a strict mathematical rule when building a composite matrix from a sequence of rotations.

The Rule:

Transformations are multiplied right-to-left. The first movement stated in the sequence is written at the far right of the mathematical equation, and subsequent movements are multiplied to the left.

Example Sequence:

1. Rotate about Z by angle alpha

2. Rotate about Y by angle beta

3. Rotate about X by angle gamma

Written Mathematically As:
ROT_comp = Rot(X, gamma) * Rot(Y, beta) * Rot(Z, alpha)

# 3. Translating Coordinate Systems

While standard kinematics uses Cartesian coordinates (X, Y, Z), robot positions can also be expressed in Cylindrical or Spherical coordinate systems by applying the composite rotation rule to a specific sequence of movements.

## A. Cylindrical Coordinate System Mapping

To reach a point (Q) using cylindrical mechanics, the sequence is:

1. Translate along X_U by 'r' units.

2. Rotate about Z_U by angle 'theta' (θ).

3. Translate along Z_U by 'z' units.

Composite Equation: 
T_comp = Trans(Z_U, z) * Rot(Z_U, θ) * Trans(X_U, r)

Multiplying these 4x4 matrices yields the standard conversion formulas:

qx = r * cos(θ)
qy = r * sin(θ)
qz = z

## B. Spherical Coordinate System Mapping

To reach the same point (Q) using spherical mechanics, the sequence is:

1. Translate along Z_U by 'r' units.

2. Rotate about Y_U by angle 'alpha' (α).

3. Rotate about Z_U by angle 'beta' (β).

Composite Equation: 
T_comp = Rot(Z_U, β) * Rot(Y_U, α) * Trans(Z_U, r)

Multiplying these matrices yields the standard spherical conversion formulas:

qx = r * sin(α) * cos(β)
qy = r * sin(α) * sin(β)
qz = r * cos(α)

# 4. Orientation in Cartesian Coordinates

In a Cartesian system, the orientation of an end-effector (like a two-finger gripper) is represented by a 3x3 rotation matrix made of three specific vectors:

Normal Vector (n)

Sliding Vector (s)

Approach Vector (a)