# Course Notes: Orientation - Roll, Pitch, Yaw & Euler Angles

# 1. Roll, Pitch, and Yaw (RPY) Orientation

The concept of Roll, Pitch, and Yaw to describe 3D orientation is derived from aeronautics and naval terminology, describing how a ship or aircraft rotates around its center axes.

Roll: Rotation about the X-axis by angle 'alpha' (α).

Pitch: Rotation about the Y-axis by angle 'beta' (β).

Yaw: Rotation about the Z-axis by angle 'gamma' (γ).

## Applying the Composite Rotation Rule to RPY:

Because all RPY rotations are taken with respect to the fixed Universal Coordinate System (U), we apply the composite rule (multiplying right-to-left):

R_B_U(rpy) = Rot(Z, γ) * Rot(Y, β) * Rot(X, α)

Multiplying these three 3x3 matrices yields the final 3x3 RPY rotation matrix.

## Extracting Angles from a Known Matrix:

If a 3x3 Cartesian orientation matrix is known (e.g., from sensor data), we can extract the RPY angles by equating elements:

α (Roll) = tan^(-1) (r_32 / r_33)

β (Pitch) = tan^(-1) (-r_31 / sqrt(r_11^2 + r_21^2))

γ (Yaw) = tan^(-1) (r_21 / r_11)

# 2. Euler Angles Orientation

Euler angles also describe 3D orientation, but with a critical difference: Rotations are taken with respect to the LOCAL Body coordinate system (B) as it moves, NOT the fixed Universal system (U).

## The Euler Sequence:

1. Rotate B about its own Z_B axis by angle 'alpha' (α).

2. Rotate B about its NEW Y_B' axis by angle 'beta' (β).

3. Rotate B about its NEWEST X_B'' axis by angle 'gamma' (γ).

## Solving for the Transformation:

Because we are rotating locally, standard matrix multiplication against the Universal frame doesn't work directly. We use a relative velocity-like concept to solve this:

Rotating the Body relative to a fixed Universe is mathematically equivalent to keeping the Body fixed and rotating the Universe by the negative angle.

We calculate the rotation of the Universe with respect to the Body (R_U_B) using the standard composite rule with negative angles:

R_U_B = Rot(X, -γ) * Rot(Y, -β) * Rot(Z, -α)

To find the final orientation of the Body with respect to the Universe (R_B_U), we take the inverse. Since it's a pure rotation matrix, the inverse is simply the transpose:

R_B_U = Transpose( R_U_B )

## Extracting Euler Angles from a Known Matrix:

α = tan^(-1) (r_21 / r_11)

β = tan^(-1) (-r_31 / sqrt(r_11^2 + r_21^2))

γ = tan^(-1) (r_32 / r_33)

# 3. Summary

A robot's position can be controlled using Cartesian, Cylindrical, or Spherical coordinate systems.

A robot's orientation can be controlled using Cartesian (Normal/Sliding/Approach vectors), Roll-Pitch-Yaw, or Euler Angles.

Modern teach-pendants allow the operator to switch between these mathematical frameworks seamlessly depending on the task geometry.