# Course Notes: Denavit-Hartenberg (DH) Notation

# 1. Introduction to DH Notation

To find the position and orientation of an end-effector with respect to the fixed base coordinate system of a serial manipulator, we must assign a coordinate frame at every joint. The standardized rules for assigning these frames and parameters were proposed by Denavit and Hartenberg in 1955.

# 2. The Four DH Parameters

The DH notation utilizes four specific parameters (two for the link structure, two for the joint relationships) to define the transformation between consecutive frames (from frame i-1 to frame i).

## Link Parameters (Defines Link Structure):

Link Length (a_i): The mutual perpendicular distance between Axis_{i-1} and Axis_i. If the two axes intersect, a_i = 0.

Angle of Twist (α_i): The angle between Axis_{i-1} and Axis_i measured about the common normal. If the axes are strictly parallel, α_i = 0.

## Joint Parameters (Defines Relative Position):

Link Offset (d_i): The distance measured along Axis_{i-1} from where a_{i-1} intersects it, to where a_i intersects it. For a prismatic (linear) joint, this is the variable.

Joint Angle (θ_i): The angle between the extension of a_{i-1} and a_i, measured about Axis_{i-1}. For a revolute (rotary) joint, this is the variable.

# 3. Rules for Coordinate Frame Assignment

Frames must be built in a strict sequence: first assign Z, then X, then calculate Y.

## Rule 1: Determining the Z-Axis

Z_i is always the axis of actuation.
If it is a revolute joint, Z is the axis about which rotation occurs. If it is a prismatic joint, Z is the axis along which translation occurs.

## Rule 2: Determining the X-Axis

The X-axis is defined based on the relationship between Z_{i-1} and Z_i:

If Parallel: X points along the common normal (mutual perpendicular distance) from Z_{i-1} to Z_i.

If Intersecting: X is chosen as either of the two remaining perpendicular directions.

If Collinear (acting along the same straight line): X can be selected anywhere in the plane perpendicular to the Z axes.

## Rule 3: Determining the Y-Axis

Y is always calculated using the right-hand rule cross product: 
Y = Z × X

# 4. Frame Transformation Sequence

To find the transformation matrix from frame i-1 to frame i (T_i^(i-1)), we follow a strict sequence of mathematical translations and rotations using the four defined parameters:

1. Rotate about Z by angle θ_i

2. Translate along Z by offset d_i

3. Rotate about X by twist angle α_i

4. Translate along X by length a_i