# Course Notes: Transformation Matrices, Screw Theory & Forward Kinematics

# 1. Deriving the Transformation Matrix

To find the transformation matrix from frame i-1 to frame i, we combine the four Denavit-Hartenberg parameters into a single sequence of transformations. The sequence relies heavily on 'Screw Theory'.

## A. The Screw Principle

A 'Screw' motion is defined as a rotation about an axis followed by a linear translation along that EXACT SAME axis (like driving a screw into a piece of wood).

Screw Z: Rotation about Z by angle θ_i followed by Translation along Z by offset d_i.

Screw X: Rotation about X by twist angle α_i followed by Translation along X by length a_i.

## B. The Final Sequence

By writing out the four respective 4x4 matrices and multiplying them in the correct composite rule sequence, we generate the final 4x4 Transformation Matrix (T_i_to_i-1).

Addressing the Composite Rule Question: 
You might notice the matrices seem to be written in reverse order (Screw Z first, then Screw X) compared to the standard composite rule. Are we violating the rule?
No. The composite rule strictly calculates T_i-1_to_i. Because our goal is to find the transformation from i to i-1, we are actually calculating the mathematical INVERSE. By taking the inverse, the order of the terms is naturally reversed, making our sequence mathematically sound.

# 2. Case Study: 2-DoF Planar Serial Manipulator

Let's apply these rules to a simple 2-Degree-of-Freedom robotic arm operating in a 2D plane (X, Y). The arm has two links (L1, L2) and two rotary (revolute) joints (θ1, θ2).

## A. Assigning the Coordinates

1. Determine Z: The axes of rotation for both joints are pointing out of the page towards you. Therefore, Z0, Z1, and Z2 are all pointing straight out of the 2D plane.

2. Determine X: Since Z0 and Z1 are parallel, the common normal (X) runs exactly along the physical length of the links (L1, L2).

3. Determine Y: Found using the right-hand rule (Z cross X).

## B. Filling the D-H Parameters Table

For Frame 1 (Joint 1 to Base):

θ1: Variable (it's a rotary motor).

d1 = 0: X0 and X1 intersect, distance along Z is zero.

α1 = 0: Z0 and Z1 are perfectly parallel.

a1 = L1: The length of the first link.

For Frame 2 (Joint 2 to Joint 1):

θ2: Variable.

d2 = 0

α2 = 0

a2 = L2: The length of the second link.

# 3. Forward Kinematics

Forward Kinematics is the process of calculating the final position (X, Y, Z) and orientation of the end-effector when the lengths of the links AND the joint angles are KNOWN.

## Calculating the 2-DoF Forward Kinematics:

To find the final position (T_2_to_Base), we multiply the transformation matrices:

T_2_Base = T_1_Base * T_2_1

When we multiply the 4x4 matrices resulting from our DH table parameters, the top-right 3x1 position vector output perfectly matches basic 2D trigonometric calculations:

qx = (L1 * cos(θ1)) + (L2 * cos(θ1 + θ2))

qy = (L1 * sin(θ1)) + (L2 * sin(θ1 + θ2))

qz = 0 (because the robot only moves on the 2D plane)

This proves that the DH notation and composite matrix rules correctly and reliably solve Forward Kinematics problems.