# Course Notes: Physical Interpretation of Edge Detection & Boundary Descriptors

# 1. Physical Interpretation of Edge Detection

In the previous section, we discussed the math behind gradient operators. Here is the physical interpretation of how those operators detect edges when scanning from a dark background to a light object and back to a dark background.

The Raw Scan: As the scanner crosses the boundary from dark to light, the light intensity value increases. When crossing back to dark, the intensity value decreases.

First-Order Derivative (Velocity equivalent): Measures the rate of change.

In solid dark or solid light zones, the rate of change is 0 (flat line).

At the edge transitioning from dark to light, there is a positive constant slope.

At the edge transitioning from light to dark, there is a negative constant slope.

Second-Order Derivative (Acceleration equivalent): Measures the rate of change of the rate of change.

At the exact moment the dark-to-light edge begins, there is a sharp positive spike. As the edge transition ends and levels out in the light zone, there is a sharp negative spike.

These sharp, distinct spikes make second-order derivatives highly effective at pinpointing exact boundary edges.

# 2. Boundary Descriptors

Once an object's boundary is found via edge detection, it must be represented mathematically so the computer can process it. This is done using Boundary Descriptors.

## A. Chain Codes:

Chain codes represent a boundary as a sequence of straight-line segments of fixed length, traveling in specific numbered directions.

4-Directional Chain Code: Uses 4 directions (0, 1, 2, 3), separated by 90 degrees.

8-Directional Chain Code: Uses 8 directions (0 through 7), separated by 45 degrees. Provides a more accurate representation of the shape.

Execution: Starting at an arbitrary point on the boundary, the computer traces the outline, recording the numbered direction of every step. The resulting sequence of numbers (e.g., 1103001122) represents the shape.

## B. Signatures:

A signature is a 1-D mathematical function representing a 2-D boundary. It plots the distance ('r') from the object's center of mass to its boundary as a function of the angle ('θ').

Circular Object: The distance 'r' is constant (the radius 'A') for all angles. The signature is a flat horizontal line: r(θ) = A.

Square Object: At θ=0, r=A (distance to the flat side). As θ rotates toward the corner (45° or π/4), the distance increases to r = A√2. This creates a repeating, non-linear peak-and-valley plot.

# 3. Object Identification

If a robot sees multiple distinct shapes, it must classify them (e.g., 'this is a chair, this is a table'). It does this by calculating a parameter called Compactness.

Compactness = (Perimeter)² / Area

Execution: The robot calculates the perimeter and area of the unidentified 2D shape on the screen to find its Compactness. It then compares this calculated value against a pre-programmed database of known compactness values for various objects to find a match.

# 4. Summary of Robot Vision Pipeline

1. Image Capture (CCD Camera)

2. Scanning & Digitizing (Electron Beam Scanner -> Analog to Digital)

3. Frame Grabbing (Creating the raw intensity matrix)

4. Pre-processing (Masking, Averaging, or Median Filtering for noise reduction)

5. Thresholding (Converting to strict black-and-white 1s and 0s)

6. Edge Detection (Applying Gradient/Laplacian operators)

7. Object Identification (Using Descriptors and Compactness matching)