# Course Notes: Topic 8 - Robot Motion Planning

# 1. Introduction to Robot Motion Planning

Motion planning is the process of determining a course of action for a robot to move from an initial position to a final goal while avoiding collisions.

Two Main Types of Motion Planning:

Gross Motion Planning (Free Space Planning): Finding a feasible, collision-free path through an environment. This is the primary focus of this module.

Fine Motion Planning (Compliant Motion Planning): Managing physical contact and force application (e.g., how much force a robotic arm must apply to write on a chalkboard without breaking the marker).

# 2. Manipulation vs. Navigation

Gross motion planning is further subdivided based on the type of robot:

Manipulation Problem: Moving a robotic arm (serial or parallel manipulator) through a workspace to reach a specific point.

Navigation Problem: Moving a mobile robot (wheeled, legged, or tracked) through an environment from start to finish.

# 3. Structured vs. Unstructured Environments

Structured Environment (The 'Find-Path' Problem): The complete layout of the environment is known beforehand. All obstacles are stationary (static). This allows for Global/Offline Planning (the 'Act-After-Thinking' process).

Unstructured Environment (Dynamic Motion Planning): The environment contains moving (dynamic) obstacles or unknown elements. This requires Local/Online Planning (the 'Act-While-Thinking' process), where the robot must constantly update its path as it moves.

# 4. Algorithmic Approaches to Motion Planning

There are two broad categories of traditional/algorithmic approaches to solving motion planning problems:

## A. Graph-Based Techniques:

These methods map the environment into a network of nodes and edges (a graph) to find a path.

Examples: Visibility Graphs, Voronoi Diagrams, Cell Decomposition, Tangent Graphs.

## B. Analytical Approaches:

These methods use mathematical fields or velocity vectors to calculate paths, often adapting to dynamic environments.

Examples: Potential Field Method, Incremental Planning, Relative Velocity Approach.

# 5. The Visibility Graph Method

Proposed by Nilsson in 1969, this was the first graph-based approach to solving the Find-Path problem in a structured, 2D environment.

## Assumptions:

The robot is treated as a single 'point'.

All obstacles are stationary, 2D polygons.

## The Algorithm:

1. Starting from the initial position (S), draw straight lines to every obstacle vertex (corner) that is in direct 'line of sight' (i.e., the line does not pass through any obstacle).

2. From those vertices, draw straight lines to all other visible vertices on any obstacle.

3. Continue this process until straight lines connect to the Goal (G).

4. This creates a web (a graph) of possible paths. The robot then selects one of these continuous sequences of lines to travel from S to G safely.

Note: The Visibility Graph generates all possible collision-free paths. To find the single fastest/shortest path (the time-optimal path), an additional optimization tool or search algorithm must be applied to the generated graph.